DIGITAL SYSTEM DESIGN LABORATORY

REPORT LAB 1

IMPLEMENTATION OF BASIC COMBINATION LOGIC CIRCUIT USING VERILOG

NAME: Bùi Gia Bảo

ID: ITITIU22019

### II. PROCEDURE

### II.1  LAB EXPERIMENT 1 :  WRITE HDL CODE TO REALIZE ALL LOGIC GATES

Dataflow model:

module allgate_DF ( a, b, yand,yor,ynot,ynand,ynor,yxor,yxnor );

input a,b;

output yand, yor, ynot, ynand, ynor, yxor, yxnor;

assign yand = a & b;		// AND Operation

assign yor = a | b;		// OR Operation

assign ynot = ~a ;		// NOT Operation

assign ynand = ~(a & b);	// NAND Operation

assign ynor = ~(a | b);		//NOR Operation

assign yxor = a ^ b;		//XOR Operation

assign yxnor =~(a^b);		//XNOR Operation

endmodule				// END of the module

Top-level verilog module in DE2-FPGA Kit

module lab1_ex1(SW,LEDG,LEDR);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

assign LEDR=SW;

allgate_DF DUT(SW[1],SW[2],LEDG[5],LEDG[4],LEDG[3],LEDG[2],LEDG[1],LEDG[0]);

endmodule

Behavior Model :

module allgate_BH ( a, b, yand,yor,ynot,ynand,ynor,yxor,yxnor );

input a,b;

output  yand, yor, ynot, ynand, ynor, yxor, yxnor;

reg  yand, yor, ynot, ynand, ynor, yxor, yxnor;

always @(*)

begin

assign yand = a & b;		// AND Operation

assign yor = a | b;		// OR Operation

assign ynot = ~a ;		// NOT Operation

assign ynand = ~(a & b);	// NAND Operation

assign ynor = ~(a | b);		//NOR Operation

assign yxor = a ^ b;		//XOR Operation

assign yxnor =~(a^b);		//XNOR Operation

end

endmodule				// END of the module

Top-level verilog module to implement the allgate_BH module in DE2-FPGA Kit

module lab1_ex1(SW,LEDG,LEDR);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

assign LEDR=SW;

allgate_BH DUT(SW[1],SW[2],LEDG[5],LEDG[4],LEDG[3],LEDG[2],LEDG[1],LEDG[0]);

endmodule

Gate-level (Structural Model)  :

module allgate_GL ( a, b, yand,yor,ynot,ynand,ynor,yxor,yxnor );

input a,b;

output yand, yor, ynot, ynand, ynor, yxor, yxnor;

and    G1(yand,a,b);		// AND Operation

or      G2(yor,a, b);		// OR Operation

not    G3(ynot,a) ;		// NOT Operation

nand G4 (ynand,a,b);		// NAND Operation

nor    G5(ynor,a,b);		//NOR Operation

xor    G6(yxor,a,b);		//XOR Operation

xnor  G7(yxnor,a,b);		//XNOR Operation

endmodule				// END of the module

Top-level verilog module to implement the allgate_GL module in DE2-FPGA Kit

module lab1_ex1(SW,LEDG,LEDR);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

assign LEDR=SW;

allgate_GL DUT(SW[1],SW[2],LEDG[5],LEDG[4],LEDG[3],LEDG[2],LEDG[1],LEDG[0]);

endmodule

Testbench Dataflow model:

`timescale 1ns/1ps

module allgate_DF_tb;

// Testbench signals

reg a, b;

wire yand, yor, ynot, ynand, ynor, yxor, yxnor;

// Instantiate the DUT

allgate_DF dut (

.a(a),

.b(b),

.yand(yand),

.yor(yor),

.ynot(ynot),

.ynand(ynand),

.ynor(ynor),

.yxor(yxor),

.yxnor(yxnor)

);

initial begin

$display("Time | a b | AND OR NOT NAND NOR XOR XNOR");

a = 0; b = 0; #5;

$display("%4t | %b %b |  %b   %b   %b    %b    %b    %b    %b",

$time, a, b, yand, yor, ynot, ynand, ynor, yxor, yxnor);

a = 0; b = 1; #5;

$display("%4t | %b %b |  %b   %b   %b    %b    %b    %b    %b",

$time, a, b, yand, yor, ynot, ynand, ynor, yxor, yxnor);

a = 1; b = 0; #5;

$display("%4t | %b %b |  %b   %b   %b    %b    %b    %b    %b",

$time, a, b, yand, yor, ynot, ynand, ynor, yxor, yxnor);

a = 1; b = 1; #5;

$display("%4t | %b %b |  %b   %b   %b    %b    %b    %b    %b",

$time, a, b, yand, yor, ynot, ynand, ynor, yxor, yxnor);

$finish;

end

endmodule

II.2 LAB EXPERIMENT 2 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT THE HALF ADDER CIRCUIT:

Dataflow model:

module half_adder_dataflow(input a, b, output s, Cout);

assign s = a ^ b;

assign Cout = a & b;

endmodule

Top level – data flow:

module lab1_ex2(SW,LEDG,LEDR);

input[3:0] SW;

output[2:0] LEDG;

output[3:0] LEDR;

assign LEDR=SW;

half_adder_dataflow DUT(SW[1],SW[2],LEDG[1],LEDG[0]);

endmodule

Behavior Model :

module half_adder_behavior(sum,carry,a,b );

output sum,carry;

input a,b;

reg sum,carry;

always @(a,b)

begin

sum   <= a ^ b;

carry <= a&b  ;

end

endmodule

Top level – Behavior Model:

module lab1_ex2 (

input  [3:0] SW,

output [2:0] LEDG,

output [3:0] LEDR

);

assign LEDR = SW;           // Mirror switches to red LEDs

// Instantiate half adder (SW[1] and SW[2] as inputs)

half_adder_behavior DUT (

.sum(LEDG[1]),

.carry(LEDG[0]),

.a(SW[1]),

.b(SW[2])

);

assign LEDG[2] = 1'b0;      // unused LEDG bit (to avoid floating output)

endmodule

Gate-level (Structural Model)  :

module half_adder_structeral(input a, b, output s, Cout);

xor G1(s,a,b);

and G2(Cout,a,b);

endmodule

Top level – Gate-level:

module lab1_ex2 (

input  [3:0] SW,

output [2:0] LEDG,

output [3:0] LEDR

);

// Display switches on the red LEDs

assign LEDR = SW;

// Instantiate structural half adder

half_adder_structeral DUT (

.a(SW[1]),

.b(SW[2]),

.s(LEDG[1]),

.Cout(LEDG[0])

);

// LEDG[2] is unused → tie to 0

assign LEDG[2] = 1'b0;

endmodule

Testbench:

`timescale 1ns/1ps

module half_adder_dataflow_tb;

reg a, b;           // Inputs to DUT

wire s, Cout;       // Outputs from DUT

// Instantiate Device Under Test

half_adder_dataflow DUT (

.a(a),

.b(b),

.s(s),

.Cout(Cout)

);

initial begin

$display("Time | a b | s Cout");

// Test 00

a = 0; b = 0; #5;

$display("%4t | %b %b | %b   %b", $time, a, b, s, Cout);

// Test 01

a = 0; b = 1; #5;

$display("%4t | %b %b | %b   %b", $time, a, b, s, Cout);

// Test 10

a = 1; b = 0; #5;

$display("%4t | %b %b | %b   %b", $time, a, b, s, Cout);

// Test 11

a = 1; b = 1; #5;

$display("%4t | %b %b | %b   %b", $time, a, b, s, Cout);

$finish;

end

endmodule

II.3 EXPERIMENT 3: WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT THE FULL ADDER CIRCUIT:

Top level Dataflow model:

module lab1_ex2 (

input  [3:0] SW,

output [2:0] LEDG,

output [3:0] LEDR

);

// Mirror switches on red LEDs

assign LEDR = SW;

// Instantiate full adder:

// Inputs: a = SW[1], b = SW[2], cin = SW[0]

// Outputs: S → LEDG[0], Cout → LEDG[1]

full_adder_DF DUT (

.a(SW[1]),

.b(SW[2]),

.cin(SW[0]),

.S(LEDG[0]),

.Cout(LEDG[1])

);

endmodule

Dataflow model:

module full_adder_DF(input a, b, cin, output S, Cout);

assign S = a ^ b ^ cin;

assign Cout = (a & b) | (b & cin) | (a & cin);

endmodule

Top level Behavior Model using case :

module lab1_ex2 (

input  [3:0] SW,

output [2:0] LEDG,

output [3:0] LEDR

);

// Show switches on RED LEDs

assign LEDR = SW;

// Instantiate FULL ADDER (behavioral)

full_adder_BH1 DUT (

.A(SW[0]),

.B(SW[1]),

.Cin(SW[2]),

.S(LEDG[1]),

.Cout(LEDG[0])

);

endmodule

Behavior Model using case :

full_adder_BH1 (input wire A, B, Cin, output reg S, output reg Cout);

always @(A or B or Cin)

begin

case (A | B | Cin)

3'b000: begin S = 0; Cout = 0; end

3'b001: begin S = 1; Cout = 0; end

3'b010: begin S = 1; Cout = 0; end

3'b011: begin S = 0; Cout = 1; end

3'b100: begin S = 1; Cout = 0; end

3'b101: begin S = 0; Cout = 1; end

3'b110: begin S = 0; Cout = 1; end

3'b111: begin S = 1; Cout = 1; end

endcase

end

Top level Behavior Model using if else :

module lab1_ex2 (

input  [3:0] SW,      // FPGA switches

output [2:0] LEDG,    // Green LEDs

output [3:0] LEDR     // Red LEDs

);

// Show switches directly on red LEDs

assign LEDR = SW;

// Instantiate FULL ADDER using SW[0], SW[1], SW[2]

full_adder DUT (

.A   (SW[0]),

.B   (SW[1]),

.Cin (SW[2]),

.S   (LEDG[0]),

.Cout(LEDG[1])

);

endmodule

Behavior Model using if else :

module full_adder( A, B, Cin, S, Cout);

input wire A, B, Cin;

output reg S, Cout;

always @(A or B or Cin)

begin

if(A==0 && B==0 && Cin==0)

begin

S=0;

Cout=0;

end

else if(A==0 && B==0 && Cin==1)

begin

S=1;

Cout=0;

end

else if(A==0 && B==1 && Cin==0)

begin

S=1;

Cout=0;

end

else if(A==0 && B==1 && Cin==1)

begin

S=0;

Cout=1;

end

else if(A==1 && B==0 && Cin==0)

begin

S=1;

Cout=0;

end

else if(A==1 && B==0 && Cin==1)

begin

S=0;

Cout=1;

end

else if(A==1 && B==1 && Cin==0)

begin

S=0;

Cout=1;

end

else if(A==1 && B==1 && Cin==1)

begin

S=1;

Cout=1;

end

end

endmodule

Top level Behavior Model:

module lab1_ex2 (

input  [3:0] SW,      // FPGA switches

output [2:0] LEDG,    // Green LEDs

output [3:0] LEDR     // Red LEDs

);

// Show switch values on red LEDs

assign LEDR = SW;

// Connect the full adder

full_adder_BH DUT (

.a(SW[2]),

.b(SW[1]),

.c(SW[0]),

.sum(LEDG[1]),

.carry(LEDG[0])

);

// Unused LEDG bit

assign LEDG[2] = 1'b0;

endmodule

Behavior Model:

module full_adder_BH(a,b,c,sum,carry);

output sum,carry;

input  a,b,c;

reg    sum,carry;

always @ (a,b,c)

begin

sum   <= a^ b^c;

carry <=(a&b) | (b&c) | (c&a);

end

endmodule

Top level Gate-level (Structural Model)  :

module lab1_ex2 (

input  [3:0] SW,

output [2:0] LEDG,

output [3:0] LEDR

);

// Show switches on red LEDs

assign LEDR = SW;

// Instantiate the full adder (structural)

full_adder_STRU DUT (

.A(SW[1]),      // First input

.B(SW[2]),      // Second input

.Cin(SW[0]),    // Carry-in

.Sum(LEDG[1]),  // Green LED shows SUM

.Carry(LEDG[0]) // Green LED shows CARRY

);

// Unused LEDG[2]

assign LEDG[2] = 1'b0;

endmodule

Gate-level (Structural Model)  :

module full_adder_STRU(a,b,cin,Cum,Carry);

output Sum,Carry;

input A,B,Cin;

wire x,y,z;

xor g1(x,A,B);

xor g2(Sum,x,Cin);

and g3(y,x,Cin);

and g4(z,A,B);

or  g5(Carry,x,y);

endmodule

II.4 EXPERIMENT 4 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT THE  RIPPLE CARRY ADDER CIRCUIT:

Top level Structural Model:

module lab1_ex2 (

input  [9:0] SW,        // use SW[7:0] as inputs

output [9:0] LEDR,      // show inputs

output [9:0] LEDG       // show adder results

);

assign LEDR = SW;       // mirror all switches to RED LEDs

// Inputs from switches

wire Cin      = SW[8];        // Carry in

wire [3:0] A  = SW[3:0];      // Lower 4 switches

wire [3:0] B  = SW[7:4];      // Upper 4 switches

// Outputs to LEDs

wire [3:0] S;

wire Cout;

// Structural 4-bit adder

four_bit_adder_STRU uut (

.cin(Cin),

.a(A),

.b(B),

.s(S),

.cout(Cout)

);

// Display result on green LEDs

assign LEDG[3:0] = S;         // Sum bits

assign LEDG[4]   = Cout;      // Final carry

assign LEDG[9:5] = 5'b0;      // unused LEDs

endmodule

Structural Model:

module full_adder_STRU(a,b,cin,Cum,Carry);

output Sum,Carry;

input A,B,Cin;

wire x,y,z;

xor g1(x,A,B);

xor g2(Sum,x,Cin);

and g3(y,x,Cin);

and g4(z,A,B);

or  g5(Carry,x,y);

endmodule

module four_bit_adder_STRU(cin,a,b,s,cout);

input [3:0] a,b;

input cin;

output [3:0] s;

output cout;

wire [2:0] w_carry;

full_adder_STRU  C1(a[0],b[0],cin,s[0], w_carry[0]);

full_adder_STRU  C2(a[1],b[1], w_carry[0],s[1], w_carry[1]);

full_adder_STRU  C3(a[2],b[2], w_carry[1],s[2], w_carry[2]);

full_adder_STRU  C4(a[3],b[3], w_carry[2],s[3], cout);

endmodule

Top level Dataflow model:

module lab1_ex2(

input  [8:0] SW,       // 9 switches: A(4), B(4), Cin

output [3:0] LEDR,     // Show sum (4 bits)

output [2:0] LEDG      // Show Cout + unused LEDs

);

wire [3:0] sum;

wire cout;

// Assign output LEDs

assign LEDR = sum;     // Show the 4-bit sum

assign LEDG[0] = cout; // Carry-out

assign LEDG[2:1] = 2'b00; // Unused Green LEDs

// Instantiate 4-bit adder

Four_bit_Adder_DF DUT (

.A(SW[3:0]),       // Lower 4 switches

.B(SW[7:4]),       // Next 4 switches

.Cin(SW[8]),       // Cin = SW8

.Sum(sum),

.Cout(cout)

);

endmodule

Dataflow model:

module Four_bit_Adder_DF(A,B,Cin,Sum,Cout);

input [3:0] A,B;

input Cin;

output wire [3:0]Sum;

output wire Cout;

wire [4:0]temp;

assign temp=A+B+Cin;

assign Sum=temp[3:0];

assign Cout=temp[4];

endmodule

II.5 EXPERIMENT 5 :WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT  2:1 MULTIPLEXER CIRCUIT:

Top level Dataflow model:

module lab1_ex2 (

input  [3:0] SW,      // SW[0]=i0, SW[1]=i1, SW[2]=sel

output [2:0] LEDG,    // LEDG[0] = mux output

output [3:0] LEDR     // show all switches on red LEDs

);

assign LEDR = SW;     // Display switch inputs

// Instantiate 2:1 multiplexer

mux21_data_flow DUT (

.i0 (SW[0]),

.i1 (SW[1]),

.sel(SW[2]),

.y  (LEDG[0])

);

// Unused green LEDs → tie them low

assign LEDG[1] = 1'b0;

assign LEDG[2] = 1'b0;

endmodule

Dataflow model:

module mux21_data_flow(i0,i1,sel,y);

input i0,i1,sel;

output y;

assign y =(i0&(~sel))|(i1&sel);

endmodule

Top level Behavior Model :

module lab1_ex2 (

input  [3:0] SW,      // SW[0] = i0, SW[1] = i1, SW[2] = sel

output [2:0] LEDG,    // LEDG[0] = output y

output [3:0] LEDR     // show all switches

);

assign LEDR = SW;     // mirror switches to red LEDs

// Instantiate MUX

mux21_Behavioural DUT (

.i0(SW[0]),

.i1(SW[1]),

.sel(SW[2]),

.y(LEDG[0])

);

// Unused LEDG[2:1] → tie them LOW

assign LEDG[1] = 1'b0;

assign LEDG[2] = 1'b0;

endmodule

Behavior Model :

module mux21_Behavioural (i0,i1,sel,y);

input i0,i1,sel;

output y;

reg y;

always@(*)

begin

if(sel==0) y=i0;

if(sel==1)y=i1;

end

endmodule

Top level Structural Model:

module lab1_ex2 (

input  [3:0] SW,

output [2:0] LEDG,

output [3:0] LEDR

);

// Show switch values on red LEDs

assign LEDR = SW;

// Instantiate 2-to-1 MUX (structural version)

mux21_structural DUT (

.i0 (SW[0]),     // Input 0

.i1 (SW[1]),     // Input 1

.sel(SW[2]),     // Selector

.y  (LEDG[0])    // MUX output

);

// Unused LEDG bits set to 0 so they do not float

assign LEDG[1] = 1'b0;

assign LEDG[2] = 1'b0;

endmodule

Structural Model:

module mux21_structural(i0,i1,sel,y);

input i0,i1,sel;

output y;

wire net1,net2,net3;

not g1(net1,sel);

and g2(net2,i1,sel);

and g3(net3,i0,net1);

or g4(y,net3,net2);

endmodule

II.6 EXPERIMENT 6 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT  4:1 MULTIPLEXER CIRCUIT:

Top level Structural Model:

module lab1_ex2 (

input  [5:0] SW,      // 6 switches

output [2:0] LEDG,    // green LEDs

output [5:0] LEDR     // red LEDs

);

// Show all switch values on LEDR

assign LEDR = SW;

// Output y from multiplexer → LEDG[0]

mux41_strutural DUT (

.i0(SW[0]),

.i1(SW[1]),

.i2(SW[2]),

.i3(SW[3]),

.s0(SW[4]),

.s1(SW[5]),

.y(LEDG[0])

);

// Unused green LEDs → force to 0

assign LEDG[1] = 1'b0;

assign LEDG[2] = 1'b0;

endmodule

Top level Structural Model:

module lab1_ex2 (

input  [7:0] SW,      // Use SW[3:0] = data inputs, SW[5:4] = select lines

output [7:0] LEDR,

output [7:0] LEDG

);

// Show all switches on red LEDs

assign LEDR = SW;

// Instantiate 4-to-1 multiplexer

mux41_strutural M1 (

.i0(SW[0]),     // data input 0

.i1(SW[1]),     // data input 1

.i2(SW[2]),     // data input 2

.i3(SW[3]),     // data input 3

.s0(SW[4]),     // select bit 0

.s1(SW[5]),     // select bit 1

.y(LEDG[0])     // output shown on LEDG[0]

);

// Unused LEDG pins → tie low

assign LEDG[7:1] = 7'b0000000;

endmodule

Structural Model:

module mux41_strutural (i0,i1,i2,i3,s0,s1,y);

input i0,i1,i2,i3,s0,s1;

output y;

wire n_s0,n_s1, a0,a1,a2,a3;

not g0(n_s0,s0);

not g1(n_s1,s1);

and g2(a0,i0,n_s0,n_s1);

and g3(a1,i1,n_s1,s0);

and g4(a2,i2,s1,n_s0);

and g5(a3,i3,s1,s0);

or   g6(y,a0,a1,a2,a3);

endmodule

Top level Dataflow model:

module lab1_ex2 (

input  [5:0] SW,      // SW[3:0] = inputs, SW[5:4] = select

output [3:0] LEDR,    // mirrors switches

output [1:0] LEDG     // outputs from muxes

);

// Mirror all switches onto red LEDs

assign LEDR = SW[3:0];

mux41_df MUX_DF (

.i0(SW[0]),

.i1(SW[1]),

.i2(SW[2]),

.i3(SW[3]),

.s0(SW[4]),

.s1(SW[5]),

.y(LEDG[0])     // DF output → LEDG[0]

);

//-------------------------

// 4:1 MUX (BEHAVIORAL version)

//-------------------------

mux41beh_v1 MUX_BEH (

.in({SW[3], SW[2], SW[1], SW[0]}),  // 4-bit input vector

.s(SW[5:4]),                        // 2-bit select

.y(LEDG[1])                         // behavioral output → LEDG[1]

);

endmodule

Dataflow model:

module mux41_df (i0,i1,i2,i3,s0,s1,y);

input i0,i1,i2,i3,s0,s1;

output y;

assign y= i0&(~s1)&(~s0) | i1 &(~s1)&s0  | i2&s1&(~s0) | i3&s1&s0;

endmodule

module mux41beh_v1(in,s,y );

output y ;

input [3:0] in ;

input [1:0] s ;

reg y;

always @ (in,s)

begin

if (s[0]==0&s[1]==0)

y <= in[0];

else if (s[0]==0&s[1]==1)

y <= in[1];

else if (s[0]==1&s[1]==0)

y <= in[2];

else

y <= in[3];

end

endmodule

Top level Behavior Model :

module lab1_ex2 (

input  [3:0] SW,      // SW[3:0] inputs

output [2:0] LEDG,    // Only LEDG[0] used for output

output [3:0] LEDR     // Mirroring switches

);

assign LEDR = SW;     // Show all switches on RED LEDs

// Instantiate 4:1 MUX (behavioral v2)

mux41beh_v2 DUT (

.in(SW[3:0]),     // 4 data inputs from switches

.s(SW[1:0]),      // select bits: SW[1:0]

.y(LEDG[0])       // output → LEDG[0]

);

// Unused LEDs tied LOW

assign LEDG[1] = 0;

assign LEDG[2] = 0;

endmodule

Behavior Model :

module mux41beh_v2(in,s,y );

output y ;

input [3:0]in ;

input [1:0]s ;

reg y;

always@(in,s)

begin

case ({s[1],s[0]})

2'b00: y <= in[0];

2'b01: y <= in[1];

2'b10: y <= in[2];

2'b11: y <= in[3];

endcase

end

endmodule

II.7 EXPERIMENT 7 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT  16:1 MULTIPLEXER CIRCUIT USING STUCTURAL MODEL FROM FIVE 4:1 MULTIPLEXER MODULE