DIGITAL SYSTEM DESIGN LABORATORY

REPORT LAB 4

BASIC BUILDING BLOCKS OF SINGLE CYCLE MICROPROCESSOR

Name: Bùi Gia Bảo

ID: ITITIU22019

### I. LAB OBJECTIVES

### This Lab experiments are intended to implement basic building blocks of Single Cycle Microprocessor

### II. DESCRIPTION

### Single Cycle Microprocessor datapath to be implemented is in figure 2.1.

### Figure 2.1: Single Cycle Microprocessor DataPath

### III. LAB PROCEDURE

### III.1 EXPERIMENT NO. 1

##### III.1.1 AIM: To implement  Program Counter

III.1.2 CODE

module Program_Counter (clk, reset, PC_in, PC_out);

input clk, reset;

input [7:0] PC_in;

output [7:0] PC_out;

reg [7:0] PC_out;

always @ (posedge clk or posedge reset)

begin

if(reset==1'b1)

PC_out<=8’b0;

else

PC_out<=PC_in;

end

endmodule

III.1.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_pc(SW,LEDG,LEDR);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

assign LEDR=SW;

Program_Counter (.clk(SW[0]), .reset(SW[1]), .PC_in(SW[9:2]), .PC_out(LEDG[7:0]));

endmodule

module Program_Counter (clk, reset, PC_in, PC_out);

input clk, reset;

input [7:0] PC_in;

output [7:0] PC_out;

reg [7:0] PC_out;

always @ (posedge clk or posedge reset)

begin

if(reset==1'b1)

PC_out<=8'b0;

else

PC_out<=PC_in;

end

endmodule

### III.2 EXPERIMENT NO. 2

##### III.2.1 AIM: To implement  32 bit Adder

III.2.2 CODE

module Adder32Bit(input1, input2, out);

input [7:0] input1, input2;

output [7:0] out;

reg [7:0]out;

always@( input1 or input2)

begin

out  <= input1 + input2;

end

endmodule

III.2.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_adder(SW,LEDG,LEDR,HEX0,HEX1,HEX2,HEX3,HEX4,HEX5);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

output[0:6] HEX0,HEX1,HEX2,HEX3,HEX4,HEX5;

wire [7:0] A,B,SUM;

assign B = SW[15:8];

assign A = SW[7:0];

Adder_8_Bit DUT(.input1(A), .input2(B), .out(SUM));

HEX_7SEG_DECODE H0(.BIN(A[3:0]),   .SSD(HEX0));

HEX_7SEG_DECODE H1(.BIN(A[7:4]),   .SSD(HEX1));

HEX_7SEG_DECODE H2(.BIN(B[3:0]),   .SSD(HEX2));

HEX_7SEG_DECODE H3(.BIN(B[7:4]),   .SSD(HEX3));

HEX_7SEG_DECODE H4(.BIN(SUM[3:0]), .SSD(HEX4));

HEX_7SEG_DECODE H5(.BIN(SUM[7:4]), .SSD(HEX5));

endmodule

module Adder_8_Bit(input1, input2, out);

input [7:0] input1, input2;

output [7:0] out;

assign	out  = input1 + input2;

endmodule

module HEX_7SEG_DECODE(BIN, SSD);

input [15:0] BIN;

output reg [0:6] SSD;

always begin

case(BIN)

0:SSD=7'b0000001;

1:SSD=7'b1001111;

2:SSD=7'b0010010;

3:SSD=7'b0000110;

4:SSD=7'b1001100;

5:SSD=7'b0100100;

6:SSD=7'b0100000;

7:SSD=7'b0001111;

8:SSD=7'b0000000;

9:SSD=7'b0001100;

10:SSD=7'b0001000;

11:SSD=7'b1100000;

12:SSD=7'b0110001;

13:SSD=7'b1000010;

14:SSD=7'b0110000;

15:SSD=7'b0111000;

endcase

end

endmodule

### III.3 EXPERIMENT NO. 3

##### III.3.1 AIM: To implement  the datapath for PC = PC + 1

III.3.2 CODE

module lab4_ex3(CLOCK_50,SW,LEDG,LEDR,HEX0,HEX1,HEX2,HEX3,HEX4,HEX5);

input	[17:0] 	SW;

input CLOCK_50;

output	[17:0] 	LEDR;

output	[7:0]	LEDG;

output	[6:0]	HEX5, HEX4, HEX3, HEX2, HEX1, HEX0;

wire clk_1Hz;

clock_divider_1Hz div1 (

.clk_in(CLOCK_50),

.reset(SW[1]),           // SW[2] = reset

.clk_out(clk_1Hz)

);

PC_add_1 DUT (.clk(clk_1Hz), .reset(SW[1]), .Instruction_Address(LEDG[7:0]));

hex7seg h4(LEDG[3:0], HEX4);

hex7seg h5(LEDG[7:4], HEX5);

endmodule

module PC_add_1 (clk, reset, Instruction_Address);

input clk, reset;

output [7:0] Instruction_Address;

wire	[7:0]	add_out;

PC pc1(.clk(clk), .reset(reset), .PC_in(add_out), .PC_out(Instruction_Address));

ADD add1(.input1(Instruction_Address), .input2(8'b0000_0001), .out(add_out));

endmodule

module hex7seg(

input  wire [3:0] hex,

output reg  [6:0] seg

);

always @(*) begin

case (hex)

4'h0: seg = 7'b1000000;

4'h1: seg = 7'b1111001;

4'h2: seg = 7'b0100100;

4'h3: seg = 7'b0110000;

4'h4: seg = 7'b0011001;

4'h5: seg = 7'b0010010;

4'h6: seg = 7'b0000010;

4'h7: seg = 7'b1111000;

4'h8: seg = 7'b0000000;

4'h9: seg = 7'b0010000;

4'hA: seg = 7'b0001000;

4'hB: seg = 7'b0000011;

4'hC: seg = 7'b1000110;

4'hD: seg = 7'b0100001;

4'hE: seg = 7'b0000110;

4'hF: seg = 7'b0001110;

default: seg = 7'b1111111;

endcase

end

endmodule

module ADD(input1, input2, out);

input [7:0] input1, input2;

output [7:0] out;

assign out = input1 + input2;

endmodule

module PC (clk, reset, PC_in, PC_out);

input clk, reset;

input [7:0] PC_in;

output [7:0] PC_out;

reg [7:0] PC_out;

always @ (posedge clk or posedge reset)

begin

if(reset==1'b1)

PC_out<=8'b0;

else

PC_out<=PC_in;

end

endmodule

module clock_divider_1Hz (

input  clk_in,

input  reset,

output reg clk_out

);

reg [25:0] count;

always @(posedge clk_in or posedge reset) begin

if (reset) begin

count   <= 26'd0;

clk_out <= 1'b0;

end else begin

if (count == 26'd24_999_999) begin

clk_out <= ~clk_out;

count   <= 26'd0;

end else begin

count <= count + 1;

end

end

end

endmodule

### III.4 EXPERIMENT NO. 4

##### III.4.1 AIM: To implement the Instruction memory

III.4.2 CODE

module Instruction_Memory (read_address, instruction, reset);

input reset;

input [9:0] read_address;

output [15:0] instruction;

reg [15:0] Imemory [1024:0];

Imemory[0] = 32'b00100000000010000000000000100000;

//addi $t0, $zero, 32

Imemory[1] = 32'b00100000000010010000000000110111;

//addi $t1, $zero, 55

Imemory[2] = 32'b00000001000010011000000000100100;

//and $s0, $t0, $t1

Imemory[3] = 32'b00000001000010011000000000100101;

//or $s0, $t0, $t1

Imemory[4] = 32'b10101100000100000000000000000100;

//sw $s0, 4($zero)

Imemory[5] = 32'b10101100000010000000000000001000;

//sw $t0, 8($zero)

Imemory[6] = 32'b00000001000010011000100000100000;

//add $s1, $t0, $t1

Imemory[7] = 32'b00000001000010011001000000100010;

//sub $s2, $t0, $t1

Imemory[8] = 32'b00010010001100100000000000001001;

//beq $s1, $s2, error0

Imemory[9] = 32'b10001100000100010000000000000100;

//lw $s1, 4($zero)

Imemory[10]= 32'b00110010001100100000000001001000;

//andi $s2, $s1, 48

Imemory[11] =32'b00010010001100100000000000001001;

//beq $s1, $s2, error1

Imemory[12] =32'b10001100000100110000000000001000;

//lw $s3, 8($zero)

Imemory[13] =32'b00010010000100110000000000001010;

//beq $s0, $s3, error2

Imemory[14] =32'b00000010010100011010000000101010;

//slt $s4, $s2, $s1 (Last)

Imemory[15] =32'b00010010100000000000000000001111;

//beq $s4, $0, EXIT

Imemory[16] =32'b00000010001000001001000000100000;

//add $s2, $s1, $0

Imemory[17] =32'b00001000000000000000000000001110;

//j Last

Imemory[18] =32'b00100000000010000000000000000000;

//addi $t0, $0, 0(error0)

Imemory[19] =32'b00100000000010010000000000000000;

//addi $t1, $0, 0

Imemory[20] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[21] =32'b00100000000010000000000000000001;

//addi $t0, $0, 1(error1)

Imemory[22] =32'b00100000000010010000000000000001;

//addi $t1, $0, 1

Imemory[23] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[24] =32'b00100000000010000000000000000010;

//addi $t0, $0, 2(error2)

Imemory[25] =32'b00100000000010010000000000000010;

//addi $t1, $0, 2

Imemory[26] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[27] =32'b00100000000010000000000000000011;

//addi $t0, $0, 3(error3)

Imemory[28] =32'b00100000000010010000000000000011;

//addi $t1, $0, 3

Imemory[29] =32'b00001000000000000000000000011111;

//j EXIT

end

endmodule

| reset | Read_address | PC_out |
| --- | --- | --- |
| 1 | 32’b0 | 32'b00100000000010000000000000100000 |
| 0 | 32’b0 | 32'b00100000000010000000000000100000 |
| 0 | 32’b0000….. 0011 | 32'b00000001000010011000000000100101 |
| 0 | 32’b0000 …….0111 | 32'b00000001000010011001000000100010 |

III.4.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_ex4_v2(SW,LEDG,LEDR,HEX0,HEX1,HEX2,HEX3,HEX4,HEX5);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

output[0:6] HEX0,HEX1,HEX2,HEX3,HEX4,HEX5;

wire[31:0] w_out_instruction;

assign LEDR[15:0] = SW[17] ? w_out_instruction[31:16] : w_out_instruction[15:0];

Instruction_Memory (.read_address(SW[5:0]), .instruction(w_out_instruction));

endmodule

module Instruction_Memory (read_address, instruction);

input [7:0] read_address;

output [31:0] instruction;

reg [31:0] Imemory [63:0];

assign instruction=Imemory[read_address];

initial begin

Imemory[0] = 32'b00100000000010000000000000100000;

//addi $t0, $zero, 32

Imemory[1] = 32'b00100000000010010000000000110111;

//addi $t1, $zero, 55

Imemory[2] = 32'b00000001000010011000000000100100;

//and $s0, $t0, $t1

Imemory[3] = 32'b00000001000010011000000000100101;

//or $s0, $t0, $t1

Imemory[4] = 32'b10101100000100000000000000000100;

//sw $s0, 4($zero)

Imemory[5] = 32'b10101100000010000000000000001000;

//sw $t0, 8($zero)

Imemory[6] = 32'b00000001000010011000100000100000;

//add $s1, $t0, $t1

Imemory[7] = 32'b00000001000010011001000000100010;

//sub $s2, $t0, $t1

Imemory[8] = 32'b00010010001100100000000000001001;

//beq $s1, $s2, error0

Imemory[9] = 32'b10001100000100010000000000000100;

//lw $s1, 4($zero)

Imemory[10]= 32'b00110010001100100000000001001000;

//andi $s2, $s1, 48

Imemory[11] =32'b00010010001100100000000000001001;

//beq $s1, $s2, error1

Imemory[12] =32'b10001100000100110000000000001000;

//lw $s3, 8($zero)

Imemory[13] =32'b00010010000100110000000000001010;

//beq $s0, $s3, error2

Imemory[14] =32'b00000010010100011010000000101010;

//slt $s4, $s2, $s1 (Last)

Imemory[15] =32'b00010010100000000000000000001111;

//beq $s4, $0, EXIT

Imemory[16] =32'b00000010001000001001000000100000;

//add $s2, $s1, $0

Imemory[17] =32'b00001000000000000000000000001110;

//j Last

Imemory[18] =32'b00100000000010000000000000000000;

//addi $t0, $0, 0(error0)

Imemory[19] =32'b00100000000010010000000000000000;

//addi $t1, $0, 0

Imemory[20] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[21] =32'b00100000000010000000000000000001;

//addi $t0, $0, 1(error1)

Imemory[22] =32'b00100000000010010000000000000001;

//addi $t1, $0, 1

Imemory[23] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[24] =32'b00100000000010000000000000000010;

//addi $t0, $0, 2(error2)

Imemory[25] =32'b00100000000010010000000000000010;

//addi $t1, $0, 2

Imemory[26] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[27] =32'b00100000000010000000000000000011;

//addi $t0, $0, 3(error3)

Imemory[28] =32'b00100000000010010000000000000011;

//addi $t1, $0, 3

Imemory[29] =32'b00001000000000000000000000011111;

//j EXIT

end

endmodule

### III.5 EXPERIMENT NO. 5

##### III.5.1 AIM: To implement the Instruction memory datapath

III.5.2 CODE

module Instruction_Datapath (clk, reset, Instruction);

input clk, reset;

output [15:0] Instruction;

endmodule

III.5.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_ex5(CLOCK_50,SW,LEDG,LEDR,HEX0,HEX1,HEX2,HEX3,HEX4,HEX5,HEX6,HEX7);

input	[17:0] 	SW;

output	[17:0] 	LEDR;

output	[7:0]	LEDG;

output	[7:0]	HEX7,HEX6,HEX5, HEX4, HEX3, HEX2, HEX1, HEX0;

input CLOCK_50;

wire clk_1Hz;

wire [31:0] OUT;

clock_divider_1Hz div1 (

.clk_in(CLOCK_50),

.reset(SW[0]),           // SW[2] = reset

.clk_out(clk_1Hz)

);

Instruction_Datapath DUT (.clk(clk_1Hz), .reset(SW[0]),.instruction_address(LEDG), .Instruction(OUT));

wire	SEL;

assign SEL = SW[17];

assign LEDR = SEL ? OUT[15:0] : OUT[31:16];

hex7seg h0(OUT[3:0], HEX0);

hex7seg h1(OUT[7:4], HEX1);

hex7seg h2(OUT[11:8], HEX2);

hex7seg h3(OUT[15:12], HEX3);

hex7seg h4(OUT[19:16], HEX4);

hex7seg h5(OUT[23:20], HEX5);

hex7seg h6(OUT[27:24], HEX6);

hex7seg h7(OUT[31:28], HEX7);

endmodule

module Instruction_Datapath (clk, reset, instruction_address, Instruction);

input clk, reset;

output [31:0] Instruction;

output	[7:0]	instruction_address;

PC_ADD DUT1(.clk(clk), .reset(reset), .Instruction_Address(instruction_address));

MEMORY DUT2(.read_address(instruction_address), .instruction(Instruction), .reset(reset));

endmodule

module hex7seg(

input  wire [3:0] hex,

output reg  [7:0] seg

);

always @(*) begin

case (hex)

4'h0: seg = 8'b11000000;

4'h1: seg = 8'b11111001;

4'h2: seg = 8'b10100100;

4'h3: seg = 8'b10110000;

4'h4: seg = 8'b10011001;

4'h5: seg = 8'b10010010;

4'h6: seg = 8'b10000010;

4'h7: seg = 8'b11111000;

4'h8: seg = 8'b10000000;

4'h9: seg = 8'b10010000;

4'hA: seg = 8'b10001000;

4'hB: seg = 8'b10000011;

4'hC: seg = 8'b11000110;

4'hD: seg = 8'b10100001;

4'hE: seg = 8'b10000110;

4'hF: seg = 8'b10001110;

default: seg = 8'b11111111;

endcase

end

endmodule

module PC_ADD (clk, reset, Instruction_Address);

input clk, reset;

output [7:0] Instruction_Address;

wire	[7:0]	add_out;

PC pc1(.clk(clk), .reset(reset), .PC_in(add_out), .PC_out(Instruction_Address));

ADD add1(.input1(Instruction_Address), .input2(8'b0000_0001), .out(add_out));

endmodule

module PC (clk, reset, PC_in, PC_out);

input clk, reset;

input [7:0] PC_in;

output [7:0] PC_out;

reg [7:0] PC_out;

always @ (posedge clk or posedge reset)

begin

if(reset==1'b1)

PC_out<=8'b0;

else

PC_out<=PC_in;

end

endmodule

module ADD(input1, input2, out);

input [7:0] input1, input2;

output [7:0] out;

assign out = input1 + input2;

endmodule

module MEMORY (read_address, instruction, reset);

input 		  reset;

input 	[7:0] read_address;

output	reg	[31:0] instruction;

reg [31:0] Imemory [1024:0];

always @(*) begin

Imemory[0] = 32'b00100000000010000000000000100000;

//addi $t0, $zero, 32

Imemory[1] = 32'b00100000000010010000000000110111;

//addi $t1, $zero, 55

Imemory[2] = 32'b00000001000010011000000000100100;

//and $s0, $t0, $t1

Imemory[3] = 32'b00000001000010011000000000100101;

//or $s0, $t0, $t1

Imemory[4] = 32'b10101100000100000000000000000100;

//sw $s0, 4($zero)

Imemory[5] = 32'b10101100000010000000000000001000;

//sw $t0, 8($zero)

Imemory[6] = 32'b00000001000010011000100000100000;

//add $s1, $t0, $t1

Imemory[7] = 32'b00000001000010011001000000100010;

//sub $s2, $t0, $t1

Imemory[8] = 32'b00010010001100100000000000001001;

//beq $s1, $s2, error0

Imemory[9] = 32'b10001100000100010000000000000100;

//lw $s1, 4($zero)

Imemory[10]= 32'b00110010001100100000000001001000;

//andi $s2, $s1, 48

Imemory[11] =32'b00010010001100100000000000001001;

//beq $s1, $s2, error1

Imemory[12] =32'b10001100000100110000000000001000;

//lw $s3, 8($zero)

Imemory[13] =32'b00010010000100110000000000001010;

//beq $s0, $s3, error2

Imemory[14] =32'b00000010010100011010000000101010;

//slt $s4, $s2, $s1 (Last)

Imemory[15] =32'b00010010100000000000000000001111;

//beq $s4, $0, EXIT

Imemory[16] =32'b00000010001000001001000000100000;

//add $s2, $s1, $0

Imemory[17] =32'b00001000000000000000000000001110;

//j Last

Imemory[18] =32'b00100000000010000000000000000000;

//addi $t0, $0, 0(error0)

Imemory[19] =32'b00100000000010010000000000000000;

//addi $t1, $0, 0

Imemory[20] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[21] =32'b00100000000010000000000000000001;

//addi $t0, $0, 1(error1)

Imemory[22] =32'b00100000000010010000000000000001;

//addi $t1, $0, 1

Imemory[23] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[24] =32'b00100000000010000000000000000010;

//addi $t0, $0, 2(error2)

Imemory[25] =32'b00100000000010010000000000000010;

//addi $t1, $0, 2

Imemory[26] =32'b00001000000000000000000000011111;

//j EXIT

Imemory[27] =32'b00100000000010000000000000000011;

//addi $t0, $0, 3(error3)

Imemory[28] =32'b00100000000010010000000000000011;

//addi $t1, $0, 3

Imemory[29] =32'b00001000000000000000000000011111;

//j EXIT

end

always @(*) begin

if(reset)

instruction = 31'b0;

else

instruction = Imemory[read_address];

end

endmodule

module clock_divider_1Hz (

input  clk_in,

input  reset,

output reg clk_out

);

reg [25:0] count;

always @(posedge clk_in or posedge reset) begin

if (reset) begin

count   <= 26'd0;

clk_out <= 1'b0;

end else begin

if (count == 26'd24_999_999) begin

clk_out <= ~clk_out;

count   <= 26'd0;

end else begin

count <= count + 1;

end

end

end

endmodule

### III.6 EXPERIMENT NO. 6

##### III.6.1 AIM: To implement the Register File

III.6.2 CODE

module Register_File (read_addr_1, read_addr_2, write_addr, read_data_1, read_data_2, write_data, RegWrite, clk, reset);

input [2:0] read_addr_1, read_addr_2, write_addr;

input [7:0] write_data;

input clk, reset, RegWrite;

output [7:0] read_data_1, read_data_2;

reg [7:0] Regfile [7:0];

integer k;

assign read_data_1 = Regfile[read_addr_1];

assign read_data_2 = Regfile[read_addr_2];

always @(posedge clk or posedge reset)

begin

if (reset==1'b1)

begin

for (k=0; k<8; k=k+1)

begin

Regfile[k] = 8'b0;

end

end

else if (RegWrite == 1'b1) Regfile[write_addr] = write_data;

end

endmodule

| reset | clk | read_addr_1 | read_data_1 | read_addr_2 | read_data_2 | RegWrite | write_addr | write_data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | x | x | 32’b0 | x | 32’b0 | x | x | x |
| 0 |  | 32’d3 | 32’b0 | 32’d4 | 32’b0 | 1 | 32’b001 | 32’b0111 |
| 0 |  | 32’d3 | 32’b0 | 32’d4 | 32’b0 | 1 | 32’b010 | 32’b1000 |
| 0 |  | 32’d3 | 32’b0 | 32’d4 | 32’b0 | 1 | 32’b011 | 32’b1001 |
| 0 |  | 32’d3 | 32’b0 | 32’d4 | 32’b0 | 1 | 32’b100 | 32’b1010 |
| 0 |  | 32’d3 | 32’b0 | 32’b00 | 32’b0 | 1 | 32’b101 | 32’b1011 |
| 0 |  | 32’b001 | 32’b0111 | 32’b101 | 32’b1011 | 0 | x | x |
| 0 |  | 32’b011 | 32’b1001 | 32’b100 | 32’b1010 | 0 | x | x |
| 0 |  | 32’b101 | 32’b1011 | 32’b101 | 32’b1011 | 0 | x | x |
| 0 |  | 32’b100 | 32’b1010 | 32’b100 | 32’b1010 | 0 | x | x |

III.6.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_ex6_v2(KEY,SW,LEDG,LEDR,HEX0,HEX1,HEX2,HEX3,HEX4,HEX5);

input[3:0] KEY;

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

output[0:6] HEX0,HEX1,HEX2,HEX3,HEX4,HEX5;

wire [7:0] W_LED_HEX_1_0,W_LED_HEX_3_2;

assign LEDR = SW;

Register_File DUT(	 .read_addr_1(SW[17:15]),

.read_addr_2(SW[14:12]),

.write_addr(SW[11:9]),

.read_data_1(W_LED_HEX_1_0),

.read_data_2(W_LED_HEX_3_2),

.write_data(SW[8:1]),

.RegWrite(SW[0]),

.clk(KEY[0]),

.reset(KEY[1]));

HEX_7SEG_DECODE H0(.BIN(W_LED_HEX_1_0[3:0]), .SSD(HEX0));

HEX_7SEG_DECODE H1(.BIN(W_LED_HEX_1_0[7:4]), .SSD(HEX1));

HEX_7SEG_DECODE H2(.BIN(W_LED_HEX_3_2[3:0]), .SSD(HEX2));

HEX_7SEG_DECODE H3(.BIN(W_LED_HEX_3_2[7:4]), .SSD(HEX3));

endmodule

module HEX_7SEG_DECODE(BIN, SSD);

input [3:0] BIN;

output reg [0:6] SSD;

always begin

case(BIN)

0:SSD=7'b0000001;

1:SSD=7'b1001111;

2:SSD=7'b0010010;

3:SSD=7'b0000110;

4:SSD=7'b1001100;

5:SSD=7'b0100100;

6:SSD=7'b0100000;

7:SSD=7'b0001111;

8:SSD=7'b0000000;

9:SSD=7'b0001100;

10:SSD=7'b0001000;

11:SSD=7'b1100000;

12:SSD=7'b0110001;

13:SSD=7'b1000010;

14:SSD=7'b0110000;

15:SSD=7'b0111000;

endcase

end

endmodule

module Register_File (read_addr_1, read_addr_2, write_addr, read_data_1, read_data_2, write_data, RegWrite, clk, reset);

input [2:0] read_addr_1, read_addr_2, write_addr;

input [7:0] write_data;

input clk, reset, RegWrite;

output [7:0] read_data_1, read_data_2;

reg [7:0] Regfile [7:0];

integer k;

assign read_data_1 = Regfile[read_addr_1];

assign read_data_2 = Regfile[read_addr_2];

always @(posedge clk or negedge reset)

begin

if (reset==1'b0)

begin

for (k=0; k<8; k=k+1)

begin

Regfile[k] = 8'b0;

end

end

else if (RegWrite == 1'b1) Regfile[write_addr] = write_data;

end

endmodule

### III.7 EXPERIMENT NO. 7

##### III.7.1 AIM: To implement the ALU

III.7.2 CODE

module alu(

input [2:0] alufn,

input [7:0] ra,

input [7:0] rb_or_imm,

output reg [7:0] aluout,

output reg zero);

parameter	ALU_OP_ADD	= 3'b000,

ALU_OP_SUB	= 3'b001,

ALU_OP_AND	= 3'b010,

ALU_OP_OR		= 3'b011,

ALU_OP_NOT_A	= 3'b100,

ALU_OP_LW		= 3'b101,

ALU_OP_SW		= 3'b110,

ALU_OP_BEQ	= 3'b111;

always @(*)

begin

case(alufn)

ALU_OP_ADD 	: aluout = ra + rb_or_imm;

ALU_OP_SUB 	: aluout = ra - rb_or_imm;

ALU_OP_AND 	: aluout = ra & rb_or_imm;

ALU_OP_OR		: aluout = ra | rb_or_imm;

ALU_OP_NOT_A	: aluout = ~ ra;

ALU_OP_LW		: aluout = ra + rb_or_imm;

ALU_OP_SW		: aluout = ra + rb_or_imm;

ALU_OP_BEQ	: begin

zero = (ra==rb_or_imm)?1'b1:1'b0;

aluout = ra - rb_or_imm;

end

endcase

end

endmodule

| alufn | ra | rb_or_imm | aluout | zero |
| --- | --- | --- | --- | --- |
| 3'b000 | 32’d8 | 32’d2 | 32’d10 | 0 |
| 3'b001 | 32’d8 | 32’d2 | 32’d6 | 0 |
| 3'b010 | 32’b1000 | 32’b0010 | 32’b0000 | 0 |
| 3'b011 | 32’b1000 | 32’b0010 | 32’b1010 | 0 |
| 3'b100 | 32’b1000 | 32’d2 | 32’b0111 | 0 |
| 3'b101 | 32’d8 | 32’d2 | 32’d10 | 0 |
| 3'b110 | 32’d8 | 32’d2 | 32’d10 | 0 |
| 3'b111 | 32’d8 | 32’d2 | 32’d6 | 0 |
| 3'b111 | 32’d8 | 32’d8 | 32’d0 | 1 |

III.7.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_ex7(LEDR, LEDG, SW);

input	[17:0] 	SW;

output	[17:0] 	LEDR;

output	[7:0]	LEDG;

assign LEDR =SW;

alu(.alufn(SW[17:15]),

.ra(SW[6:0]),

.rb_or_imm(SW[14:7]),

.aluout(LEDG[6:0]),

.zero(LEDG[7]));

endmodule

### III.8 EXPERIMENT NO. 8

##### III.8.1 AIM: To implement the 32 bit RAM

III.8.2 CODE

module Data_Memory (addr, write_data, read_data, clk, reset, MemRead, MemWrite);

input [7:0] addr;

input [7:0] write_data;

output [7:0] read_data;

input clk, reset, MemRead, MemWrite;

reg [7:0] DMemory [7:0];

integer k;

assign read_data = (MemRead) ? DMemory[addr] : 8'bx;

always @(posedge clk or posedge reset)

begin

if (reset == 1'b1)

begin

for (k=0; k<8; k=k+1)

begin

DMemory[k] = 8'b0;

end

end

else

if (MemWrite) DMemory[addr] = write_data;

end

endmodule

III.8.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_ex8(SW,LEDG,LEDR);

input	[17:0] 	SW;

output	[17:0] 	LEDR;

output	[7:0]	LEDG;

assign LEDR = SW;

Data_Memory (	.addr(SW[6:0]),

.write_data(SW[13:7]),

.read_data(LEDG[6:0]),

.clk(SW[17]),

.reset(SW[16]),

.MemRead(SW[15]),

.MemWrite(SW[14]));

Endmodule

### III.9 EXPERIMENT NO. 9

##### III.9.1 AIM: To implement  Multiplexer

III.9.2 CODE

module Mux_N_bit (in0, in1, mux_out, select);

parameter N = 32;

input [N-1:0] in0, in1;

output [N-1:0] mux_out;

input control;

assign mux_out = select? in1: in0 ;

endmodule

III.9.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_ex9(SW, LEDR, LEDG);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

Mux_N_bit (.in0(SW[7:0]), .in1(SW[15:8]), .mux_out(LEDG[7:0]), .select(SW[16]));

endmodule

module Mux_N_bit (in0, in1, mux_out, select);

parameter N = 8;

input select;

input [N-1:0] in0, in1;

output [N-1:0] mux_out;

assign mux_out = select? in1: in0 ;

endmodule

### III.10 EXPERIMENT NO. 10

##### III.10.1 AIM: To implement  Sign_Extension

III.10.2 CODE

module Sign_Extension (sign_in, sign_out);

input [15:0] sign_in;

output [31:0] sign_out;

assign sign_out[15:0]=sign_in[15:0];

assign sign_out[31:16]=sign_in[15]?{16{1'b1}}:16'b0;

endmodule

III.10.3 LAB ASSIGNMENT
1. Write testbenches to verify above module and attach waveforms.

2. Write the Top level module to implement this module in FPGA KIT

module lab4_ex10(SW, LEDR,LEDG);

input[17:0] SW;

output[7:0] LEDG;

output[17:0] LEDR;

Sign_Extension (.sign_in(SW[4:0]), .sign_out(LEDG[7:0]));

endmodule

module Sign_Extension (sign_in, sign_out);

input [4:0] sign_in;

output [7:0] sign_out;

assign sign_out[4:0]=sign_in[4:0];

assign sign_out[7:5]=sign_in[4]?{3{1'b1}}:3'b0;

endmodule