DIGITAL SYSTEM DESIGN LABORATORY

REPORT LAB 5

SINGLE CYCLE MICROPROCESSOR DATAPATH DESIGN

Name: Bùi Gia Bảo

ID: ITITIU22019

### I. LAB OBJECTIVES

### This Lab experiments are intended to design and test a Single Cycle Microprocessor

### II. DESCRIPTION

### Single Cycle Microprocessor datapath to be implemented is in figure 2.1.

### Figure 2.1: Single Cycle Microprocessor DataPath

### III. LAB PROCEDURE

### III.1 EXPERIMENT NO. 1

##### III.1.1 AIM: To understand and write the assembly code using MIPS Instruction set

##### register number of MIPS compiler conventions

##### MIPS Assembly Language Sumarize:

##### Operation code (Op code) sumarize:

ALU opcode and Function

##### Instruction Formats

III.1.2 CODE

a) Assembly Code sample 1:

Instruction			      Meaning

addi $s0, $zero, 33      	load immediate value 33 to register $s0

addi $s1, $zero, 66      	load immediate value 66 to register $s1

add  $s2, $s0, $s1       	$s2 = $s0 + $s1
sub  $s3, $s1, $s0	   	$s1 = $s1 – $s0
sw $s3, 10($s2)	   	Memory[$s2+10] = $s3

lw $s1, 10($s2)	   	$s1 = Memory[$s2+10]

b) Assembly Code sample 2:

Assume the code start from address PC=0x00000000, one instruction is store in one memory location.

Instruction		  	Meaning

addi $s2, $zero, 55      	load immediate value 55 to register $S2

addi $s3, $zero, 22      	load immediate value 22 to register $S3

addi $s5, $zero, 33      	load immediate value 55 to register $S3

add $s4,$s2,$s3	 	$s4 = $s2 + $s3
sub $s1,$s2,$s3	 	$s1 = $s2 – $s3
sw $s1,100($s2)		Memory[$s2+100] = $s1

lw $s1,100($s2)		$s1 = Memory[$s2+100] 
bne $s1,$s5,End	 	Next instr. is at End if $s4 !=$s5

addi $s6, $zero, 10        	load immediate value 10 to register $s6
beq $s4,$s5, End	 	Next instr. is at End if $s4 = $s5

addi $s6, $zero, 20        	load immediate value 20 to register $s6

End: 	j End		 		jump Here

III.1.3 LAB ASSIGNMENT
a) Compile the Assembly Assembly Code sample 1 into machine code (decimal code and binary code)

b) Explain briefly the meaning of Assembly Code sample 1

c) Compile the Assembly Assembly Code sample 2 into machine code (decimal code and binary code)

d) Explain briefly the meaning of Assembly Code sample 2

### III.2 EXPERIMENT NO. 2

##### III.2.1 AIM: To implement  R-Type Datapath

##### Given the assembly code

addi $s1, $zero, 33      	load immediate value 33 to register $s0

addi $s2, $zero, 66      	load immediate value 66 to register $s1

##### add  $s0, $s1, $s2       	$s0 = $s1 + $s2

##### Translate into Macchine code:

##### Modify the code register file:

##### Assign initial value of the register 17=33 ( $s1=33)

##### Assign initial value of the register 18=33 ( $s2=66)

III.2.2 CODE

module lab5_ex2(KEY,SW,LEDG,LEDR,HEX0,HEX1,HEX2,HEX3,HEX4,HEX5,HEX6,HEX7);

input[3:0] KEY;

input	[17:0] 	SW;

output	[17:0] 	LEDR;

output	[7:0]	LEDG;

output [0:6] HEX0,HEX1,HEX2,HEX3,HEX4,HEX5,HEX6,HEX7;

wire [31:0] w_hex;

assign LEDR =SW;

wire [2:0] op;

//assign op = SW[17] ? 3'b010 : 3'b110;

DatapathR_Type_Add(.rs(SW[14:10]), .rt(SW[9:5]), .rd(SW[4:0]),

.ALUop(SW[17:15]),

.Zero(LEDG[7]),

.ALU_out(w_hex),

.reset(KEY[0]),

.clk(KEY[1]),

.regWrite(1'b1));

HEX_7SEG_DECODE H0(.BIN(w_hex[3:0]), .SSD(HEX0));

HEX_7SEG_DECODE H1(.BIN(w_hex[7:4]), .SSD(HEX1));

HEX_7SEG_DECODE H2(.BIN(w_hex[11:8]), .SSD(HEX2));

HEX_7SEG_DECODE H3(.BIN(w_hex[15:12]), .SSD(HEX3));

HEX_7SEG_DECODE H4(.BIN(w_hex[19:16]), .SSD(HEX4));

HEX_7SEG_DECODE H5(.BIN(w_hex[23:20]), .SSD(HEX5));

HEX_7SEG_DECODE H6(.BIN(w_hex[27:24]), .SSD(HEX6));

HEX_7SEG_DECODE H7(.BIN(w_hex[31:28]), .SSD(HEX7));

endmodule

module DatapathR_Type_Add(rs, rt, rd,ALUop, Zero ,ALU_out, reset, clk, regWrite);

input[4:0] rs, rt, rd;

input[2:0] ALUop;

output Zero;

output[31:0] ALU_out;

wire[31:0] w_rd_1, w_rd_2, w_rd_11, w_rd_22;

input clk;

input reset;

input regWrite;

assign w_rd_11 = 32'h0000_0003;

assign w_rd_22 = 32'h0000_0001;

Register_File ( .read_addr_1(rs),

.read_addr_2(rt),

.write_addr(rd),

.read_data_1(w_rd_1),

.read_data_2(w_rd_2),

.write_data(ALU_out),

.RegWrite(regWrite),

.clk(clk),

.reset(reset));

alu( 	.alufn(ALUop),

.ra(w_rd_1),

.rb_or_imm(w_rd_2),

.aluout(ALU_out),

.zero(Zero));

endmodule

module HEX_7SEG_DECODE(BIN, SSD);

input [3:0] BIN;

output reg [0:6] SSD;

always begin

case(BIN)

0:SSD=7'b0000001;

1:SSD=7'b1001111;

2:SSD=7'b0010010;

3:SSD=7'b0000110;

4:SSD=7'b1001100;

5:SSD=7'b0100100;

6:SSD=7'b0100000;

7:SSD=7'b0001111;

8:SSD=7'b0000000;

9:SSD=7'b0001100;

10:SSD=7'b0001000;

11:SSD=7'b1100000;

12:SSD=7'b0110001;

13:SSD=7'b1000010;

14:SSD=7'b0110000;

15:SSD=7'b0111000;

endcase

end

endmodule

module Register_File (read_addr_1, read_addr_2, write_addr, read_data_1, read_data_2, write_data, RegWrite, clk, reset);

input [4:0] read_addr_1, read_addr_2, write_addr;

input [31:0] write_data;

input clk, reset, RegWrite;

output [31:0] read_data_1, read_data_2;

reg [31:0] Regfile [31:0];

integer k;

//assign Regfile[17] = 32'h33;

//assign Regfile[18] = 32'h66;

assign read_data_1 = RegWrite ? Regfile[read_addr_1] : 32'b0;

assign read_data_2 = RegWrite ? Regfile[read_addr_2] : 32'b0;

always @(posedge clk or negedge reset)

begin

if (reset==1'b0)

begin

for (k=0; k<32; k=k+1)

begin

Regfile[k] = 32'b0;

end

end

else begin

if (RegWrite == 1'b1) Regfile[write_addr] = write_data;

Regfile[17] = 32'h33;

Regfile[18] = 32'h66;

end

end

endmodule

module alu(

input [2:0] alufn,

input [31:0] ra,

input [31:0] rb_or_imm,

output reg [31:0] aluout,

output wire zero);

parameter	ALU_OP_ADD	= 3'b010,

ALU_OP_SUB	= 3'b110,

ALU_OP_AND	= 3'b000,

ALU_OP_OR		= 3'b001,

ALU_OP_NOT_A	= 3'b100,

ALU_OP_LW		= 3'b101,

ALU_OP_SW		= 3'b011,

ALU_OP_BEQ	= 3'b111;

assign  zero = ((alufn == ALU_OP_BEQ) && (ra==rb_or_imm))?1'b1:1'b0;

always @(*)

begin

case(alufn)

ALU_OP_ADD 	: aluout = ra + rb_or_imm;

ALU_OP_SUB 	: aluout = ra - rb_or_imm;

ALU_OP_AND 	: aluout = ra & rb_or_imm;

ALU_OP_OR		: aluout = ra | rb_or_imm;

ALU_OP_NOT_A	: aluout = ~ ra;

ALU_OP_LW		: aluout = ra + rb_or_imm;

ALU_OP_SW		: aluout = ra + rb_or_imm;

ALU_OP_BEQ	: aluout = ra - rb_or_imm;

endcase

end

endmodule

### III.3 EXPERIMENT NO. 3

##### III.3.1 AIM: To implement SW  I-Type Instruction  Datapath

III.3.2 CODE

module lab5_ex3(KEY,SW,LEDG,LEDR,HEX0,HEX1,HEX2);

input[3:0] KEY;

input	[17:0] 	SW;

output	[17:0] 	LEDR;

output	[7:0]	LEDG;

output [0:6] HEX0,HEX1,HEX2;

wire [7:0] w_hex,w_hex_2;

assign LEDR =SW;

//assign op = SW[17] ? 3'b010 : 3'b110;

SW_datapath (	.rs(SW[2:0]),

.rt(SW[5:3]),

.offset(SW[10:6]),

.ALUop(SW[13:11]),

.MemWrite(SW[14]),

.MemRead(SW[15]),

.Mem_out(w_hex),

.reset(SW[17]),

.clk(KEY[1]),

.regWrite(1'b1));

HEX_7SEG_DECODE H0(.BIN(w_hex[3:0]), .SSD(HEX0));

HEX_7SEG_DECODE H1(.BIN(w_hex[7:4]), .SSD(HEX1));

endmodule

module SW_datapath (rs, rt, offset,ALUop,MemWrite,MemRead,Mem_out, reset, clk, regWrite);

input[2:0] rs, rt;

input[2:0] ALUop;

input[4:0] offset;

input MemWrite, MemRead;

output[7:0] Mem_out;

wire[7:0] ALU_out;

wire[7:0] w_rd_1, w_rd_2, w_rd_11, w_rd_22;

wire[7:0] w_mrd, w_offset_ext;

input clk;

input reset;

input regWrite;

Register_File ( 	.read_addr_1(rs),

.read_addr_2(rt),

.write_addr(rt),

.read_data_1(w_rd_1),

.read_data_2(w_rd_2),

.write_data(8'b00000000),

.RegWrite(regWrite),

.clk(clk),

.reset(reset));

alu( 	.alufn(ALUop),

.ra(w_rd_1),

.rb_or_imm(w_offset_ext),

.aluout(ALU_out),

.zero(Zero));

Data_Memory (	.addr(ALU_out),

.write_data(w_rd_2),

.read_data(Mem_out),

.clk(clk),

.reset(reset),

.MemRead(MemRead),

.MemWrite(MemWrite));

Sign_Extension (.sign_in(offset), .sign_out(w_offset_ext));

endmodule

module HEX_7SEG_DECODE(BIN, SSD);

input [3:0] BIN;

output reg [0:6] SSD;

always begin

case(BIN)

0:SSD=7'b0000001;

1:SSD=7'b1001111;

2:SSD=7'b0010010;

3:SSD=7'b0000110;

4:SSD=7'b1001100;

5:SSD=7'b0100100;

6:SSD=7'b0100000;

7:SSD=7'b0001111;

8:SSD=7'b0000000;

9:SSD=7'b0001100;

10:SSD=7'b0001000;

11:SSD=7'b1100000;

12:SSD=7'b0110001;

13:SSD=7'b1000010;

14:SSD=7'b0110000;

15:SSD=7'b0111000;

endcase

end

endmodule

module Register_File (read_addr_1, read_addr_2, write_addr, read_data_1, read_data_2, write_data, RegWrite, clk, reset);

input [2:0] read_addr_1, read_addr_2, write_addr;

input [7:0] write_data;

input clk, reset, RegWrite;

output [7:0] read_data_1, read_data_2;

reg [7:0] Regfile [7:0];

integer k;

assign read_data_1 = RegWrite ? Regfile[read_addr_1] : 8'b0;

assign read_data_2 = RegWrite ? Regfile[read_addr_2] : 8'b0;

always @(posedge clk or negedge reset)

begin

if (reset==1'b0)

begin

for (k=0; k<8; k=k+1)

begin

Regfile[k] = 8'b0;

end

end

else begin

if (RegWrite == 1'b1) Regfile[write_addr] = write_data;

Regfile[3] = 8'h1;

Regfile[5] = 8'h2;

Regfile[6] = 8'h3;

Regfile[4] = 8'h2;

end

end

endmodule

module alu(

input [2:0] alufn,

input [7:0] ra,

input [7:0] rb_or_imm,

output reg [7:0] aluout,

output wire zero);

parameter	ALU_OP_ADD	= 3'b010,

ALU_OP_SUB	= 3'b110,

ALU_OP_AND	= 3'b000,

ALU_OP_OR		= 3'b001,

ALU_OP_NOT_A	= 3'b100,

ALU_OP_LW		= 3'b101,

ALU_OP_SW		= 3'b011,

ALU_OP_BEQ	= 3'b111;

assign  zero = ((alufn == ALU_OP_BEQ) && (ra==rb_or_imm))?1'b1:1'b0;

always @(*)

begin

case(alufn)

ALU_OP_ADD 	: aluout = ra + rb_or_imm;

ALU_OP_SUB 	: aluout = ra - rb_or_imm;

ALU_OP_AND 	: aluout = ra & rb_or_imm;

ALU_OP_OR		: aluout = ra | rb_or_imm;

ALU_OP_NOT_A	: aluout = ~ ra;

ALU_OP_LW		: aluout = ra + rb_or_imm;

ALU_OP_SW		: aluout = ra + rb_or_imm;

ALU_OP_BEQ	: aluout = ra - rb_or_imm;

endcase

end

endmodule

module Data_Memory (addr, write_data, read_data, clk, reset, MemRead, MemWrite);

input [7:0] addr;

input [7:0] write_data;

output [7:0] read_data;

input clk, reset, MemRead, MemWrite;

reg [7:0] DMemory [7:0];

integer k;

assign read_data = (MemRead && ~MemWrite) ? DMemory[addr] : 8'bx;

always @(posedge clk or negedge reset)

begin

if (reset == 1'b0)

begin

for (k=0; k<8; k=k+1)

begin

DMemory[k] = 8'b0;

end

end

else

if (MemWrite) DMemory[addr] = write_data;

end

endmodule

module Sign_Extension (sign_in, sign_out);

input [4:0] sign_in;

output [7:0] sign_out;

assign sign_out[4:0]=sign_in[4:0];

assign sign_out[7:5]=sign_in[4]?{3{1'b1}}:3'b0;

endmodule

### III.4 EXPERIMENT NO. 4

##### III.4.1 AIM: To implement LW  I-Type Instruction  Datapath

III.4.2 CODE

module lab5_ex4(KEY,SW,LEDG,LEDR,HEX0,HEX1,HEX2);

input[3:0] KEY;

input	[17:0] 	SW;

output	[17:0] 	LEDR;

output	[7:0]	LEDG;

output [0:6] HEX0,HEX1,HEX2;

wire [7:0] w_hex,w_hex_2;

assign LEDR =SW;

LW_datapath (	.rs(SW[2:0]),

.rt(SW[5:3]),

.offset(SW[10:6]),

.ALUop(SW[13:11]),

.MemWrite(SW[14]),

.MemRead(SW[15]),

.LW_out(w_hex),

.reset(SW[17]),

.clk(KEY[1]),

.regWrite(1'b1));

HEX_7SEG_DECODE H0(.BIN(w_hex[3:0]), .SSD(HEX0));

HEX_7SEG_DECODE H1(.BIN(w_hex[7:4]), .SSD(HEX1));

endmodule

module LW_datapath (rs, rt, offset, ALUop,MemWrite,MemRead, LW_out, reset, clk, regWrite);

input[2:0] rs, rt;

input[2:0] ALUop;

input[4:0] offset;

input MemWrite, MemRead;

output[7:0] LW_out;

wire[7:0] ALU_out;

wire[7:0] w_rd_1, w_rd_2, w_rd_11, w_rd_22;

wire[7:0] w_mrd, w_offset_ext;

input clk;

input reset;

input regWrite;

Register_File ( 	.read_addr_1(rs),

.read_addr_2(rt),

.write_addr(rt),

.read_data_1(w_rd_1),

.read_data_2(w_rd_2),

.write_data(LW_out),

.RegWrite(regWrite),

.clk(clk),

.reset(reset));

alu( 	.alufn(ALUop),

.ra(w_rd_1),

.rb_or_imm(w_offset_ext),

.aluout(ALU_out),

.zero(Zero));

Data_Memory (	.addr(ALU_out),

.write_data(8'b0000000),

.read_data(LW_out),

.clk(clk),

.reset(reset),

.MemRead(MemRead),

.MemWrite(MemWrite));

Sign_Extension (.sign_in(offset), .sign_out(w_offset_ext));

endmodule

module HEX_7SEG_DECODE(BIN, SSD);

input [3:0] BIN;

output reg [0:6] SSD;

always begin

case(BIN)

0:SSD=7'b0000001;

1:SSD=7'b1001111;

2:SSD=7'b0010010;

3:SSD=7'b0000110;

4:SSD=7'b1001100;

5:SSD=7'b0100100;

6:SSD=7'b0100000;

7:SSD=7'b0001111;

8:SSD=7'b0000000;

9:SSD=7'b0001100;

10:SSD=7'b0001000;

11:SSD=7'b1100000;

12:SSD=7'b0110001;

13:SSD=7'b1000010;

14:SSD=7'b0110000;

15:SSD=7'b0111000;

endcase

end

endmodule

module Register_File (read_addr_1, read_addr_2, write_addr, read_data_1, read_data_2, write_data, RegWrite, clk, reset);

input [2:0] read_addr_1, read_addr_2, write_addr;

input [7:0] write_data;

input clk, reset, RegWrite;

output [7:0] read_data_1, read_data_2;

reg [7:0] Regfile [7:0];

integer k;

assign read_data_1 = RegWrite ? Regfile[read_addr_1] : 8'b0;

assign read_data_2 = RegWrite ? Regfile[read_addr_2] : 8'b0;

always @(posedge clk or negedge reset)

begin

if (reset==1'b0)

begin

for (k=0; k<8; k=k+1)

begin

Regfile[k] = 8'b0;

end

end

else begin

if (RegWrite == 1'b1) Regfile[write_addr] = write_data;

end

end

endmodule

module alu(

input [2:0] alufn,

input [7:0] ra,

input [7:0] rb_or_imm,

output reg [7:0] aluout,

output wire zero);

parameter	ALU_OP_ADD	= 3'b010,

ALU_OP_SUB	= 3'b110,

ALU_OP_AND	= 3'b000,

ALU_OP_OR		= 3'b001,

ALU_OP_NOT_A	= 3'b100,

ALU_OP_LW		= 3'b101,

ALU_OP_SW		= 3'b011,

ALU_OP_BEQ	= 3'b111;

assign  zero = ((alufn == ALU_OP_BEQ) && (ra==rb_or_imm))?1'b1:1'b0;

always @(*)

begin

case(alufn)

ALU_OP_ADD 	: aluout = ra + rb_or_imm;

ALU_OP_SUB 	: aluout = ra - rb_or_imm;

ALU_OP_AND 	: aluout = ra & rb_or_imm;

ALU_OP_OR		: aluout = ra | rb_or_imm;

ALU_OP_NOT_A	: aluout = ~ ra;

ALU_OP_LW		: aluout = ra + rb_or_imm;

ALU_OP_SW		: aluout = ra + rb_or_imm;

ALU_OP_BEQ	: aluout = ra - rb_or_imm;

endcase

end

endmodule

module Data_Memory (addr, write_data, read_data, clk, reset, MemRead, MemWrite);

input [7:0] addr;

input [7:0] write_data;

output [7:0] read_data;

input clk, reset, MemRead, MemWrite;

reg [7:0] DMemory [7:0];

integer k;

assign read_data = (MemRead && ~MemWrite) ? DMemory[addr] : 8'bx;

always @(posedge clk or negedge reset)

begin

if (reset == 1'b0)

begin

for (k=0; k<8; k=k+1)

begin

DMemory[k] = 8'b0;

end

end

else begin

if (MemWrite) DMemory[addr] = write_data;

DMemory[5] = 8'h8;

DMemory[6] = 8'h99;

DMemory[7] = 8'h55;

end

end

endmodule

module Sign_Extension (sign_in, sign_out);

input [4:0] sign_in;

output [7:0] sign_out;

assign sign_out[4:0]=sign_in[4:0];

assign sign_out[7:5]=sign_in[4]?{3{1'b1}}:3'b0;

endmodule

### III.5 EXPERIMENT NO. 5

##### III.5.1 AIM: To implement I-Type Instruction Beq  datapath

III.5.2 CODE

module beq_Datapath (rs,rt,offset,ALUOp,PC_plus_4, Zero,beq_add)

endmodule

III.5.3 LAB ASSIGNMENT
1) Write Verilog code to implement beq_Datapath module

2) Write testbenches to verify beq_Datapath module, simulate and verify the output data.

III.10.3 LAB ASSIGNMENT
Write testbenches to verify above blocks and attach waveforms.

III. LAB REPORT GUIDELINES

Students write up a report on the Verilog HDL implementation experiment projects created in this lab. The lab report should include Verilog code for the module under test, Verilog test bench code and a truth table results, and example data input and output to validate the experiment. Simulation Result in form of Simulation Capture Screen.